THE UNIVERSITY OF ZAMBIA

SCHOOL OF NATURAL SCIENCES

DEPARTMENT OF COMPUTER SCIENCE

INTELIBIN - SMART WASTE MANAGEMENT SYSTEM

BY

BAUTIS CHILESHE

2022036565

CSC 4004 FINAL YEAR COURSE PROJECT

A THESIS SUBMITTED IN PARTIAL FULFILMENT OF THE REQUIREMENTS FOR THE AWARD OF A BACHELOR'S DEGREE IN COMPUTER SCIENCE

SUPERVISOR: Mrs KABEMBA

2026

# ABSTRACT

Waste-bin monitoring can depend on manual inspection, while conventional bin lids may require direct physical contact. This project presents InteliBin, a Smart Waste Management System that integrates embedded sensing, automatic lid control, serial communication, data processing, storage and a web-based monitoring interface. The prototype uses an Arduino Uno, two HC-SR04 ultrasonic sensors and a servo motor. One ultrasonic sensor detects a person approaching the bin at an approximate threshold of 15 cm. When a person is detected, the servo opens the lid to 180 degrees, keeps it open for approximately five seconds and returns it to the closed position of 90 degrees. The control logic waits for the person to leave before permitting another opening event.

A second HC-SR04 sensor is directed into the bin to estimate waste fill level. Seven distance readings are collected and a median-based approach is used to reduce the influence of noisy measurements. Calibration was established using 20.9 cm as the empty-bin distance and 4.0 cm as the full-bin distance. The measured distance is converted into a percentage between 0% and 100%. Sensor readings and system status are communicated from the Arduino to a personal computer through USB serial communication, after which PC-side software integrates the embedded subsystem with a backend/API, database and web dashboard.

The project demonstrates a practical integration of embedded hardware and software for smart-bin monitoring. Details verified from the source code show that InteliBin is implemented with Arduino C/C++ firmware, a Python serial-to-MySQL bridge, PHP/PDO backend and API files, a MySQL/MariaDB database, and a browser-based dashboard.

# DECLARATION

I, BAUTIS CHILESHE, declare that the work presented in this thesis, entitled InteliBin - Smart Waste Management System, is my own work except where sources have been acknowledged. This thesis has not been submitted for the award of a degree at another institution.

Author Signature	: ____________________________
Date			: ____________________________

Supervisor		: ____________________________
Supervisor Signature	: ____________________________

# DEDICATION

No dedication text is specified in the repository source code.

# ACKNOWLEDGEMENTS

No acknowledgement text is specified in the repository source code.

Table of Contents

ABSTRACT	i

DECLARATION	ii

DEDICATION	iii

ACKNOWLEDGEMENTS	iv

LIST OF FIGURES	xi

LIST OF TABLES	xii

1. INTRODUCTION	1

1.1 Background	1

1.2 Problem Statement	2

1.3 Research Questions	3

1.4 Aim	3

1.5 Objectives	3

1.6 Scope	4

2. LITERATURE REVIEW	5

2.1 Introduction	5

2.2 Smart Waste Management and IoT	5

2.3 Smart Bins and Fill-Level Monitoring	5

2.4 Ultrasonic Sensing for Waste-Level Detection	6

2.5 Automated Lid Systems	6

2.6 Review of Existing Approaches	6

2.7 Development Gap	7

2.8 Research Methodology	7

2.8.1 Background Research	8

2.8.2 System Analysis	8

2.8.3 Iterative Development and Testing	8

2.9 Summary	8

3. REQUIREMENTS ANALYSIS	9

3.1 Development Methodology	9

3.2 Required Resources	9

3.3 Functional Requirements	10

3.4 Non-Functional Requirements	10

4. SYSTEM DESIGN	11

4.1 Proposed System	11

4.2 Hardware Design	11

4.3 Person Detection and Lid-Control Design	11

4.4 Fill-Level Measurement Design	12

4.5 Software Architecture	12

4.6 Data Flow	13

4.7 Database Design	13

4.8 Dashboard Design	13

5. SYSTEM IMPLEMENTATION	14

5.1 Description of the Developed System	14

5.2 Hardware Implementation	14

5.2.1 Arduino Uno	14

5.2.2 Person-Detection Sensor	14

5.2.3 Fill-Level Sensor	14

5.2.4 Servo and Lid Mechanism	14

5.2.5 Physical Integration	15

5.3 Embedded Software Implementation	15

5.3.1 Distance Measurement	15

5.3.2 Person Detection and Leave Logic	15

5.3.3 Median Filtering	15

5.3.4 Calibration and Fill Percentage	15

5.3.5 Serial Output	16

5.4 Software Integration	16

5.4.1 PC Serial Listener	16

5.4.2 Backend/API	16

5.4.3 Database	16

5.4.4 Web Dashboard	16

5.5 Complete System Integration	17

6. TESTING AND VERIFICATION	18

6.1 Testing Methodology	18

6.2 Component Testing	18

6.3 Integration Testing	18

6.4 System Testing	19

6.5 Analysis of Test Results	19

6.6 Objective-Implementation-Verification Mapping	20

7. CONCLUSION	21

7.1 Results and Achievements	21

7.2 Limitations and Problems Faced	21

7.3 Future Work and Recommendations	22

7.4 Conclusion	22

8. APPENDICES	23

Appendix A: Arduino Source Code	23

Appendix B: Hardware Wiring and Pin Configuration	23

Appendix C: Software Installation and Deployment	23

Appendix D: Database Schema	23

Appendix E: API and Serial Communication Examples	23

Appendix F: Selected Backend Code	23

Appendix G: Additional Test Evidence	23

9. REFERENCES	25

# LIST OF FIGURES

Figure 1: InteliBin Overall System Architecture

Figure 2: InteliBin Hardware Wiring Diagram

Figure 3: Automatic Lid Control Flowchart

Figure 4: Fill-Level Measurement Flowchart

Figure 5: InteliBin Data Flow Diagram

Figure 6: Database Entity Relationship Diagram

Figure 7: Completed InteliBin Prototype

Figure 8: Web Dashboard

Figure 9: Fill-Level Monitoring Interface

Figure 10: Sensor/Lid Status Interface

# LIST OF TABLES

Table 1: Comparison of Selected Smart Waste Approaches

Table 2: InteliBin Hardware Components

Table 3: Arduino Pin Configuration

Table 4: Functional Requirements

Table 5: Non-Functional Requirements

Table 6: Fill-Level Calibration Parameters

Table 7: Component Test Cases

Table 8: Integration Test Cases

Table 9: System Test Cases

Table 10: Objective-Implementation-Verification Matrix

# 1. INTRODUCTION

## 1.1 Background

Waste management is an important service for communities, institutions and cities. The United Nations Environment Programme (UNEP) reports that global municipal solid waste generation is expected to increase substantially toward 2050, reinforcing the need for improved waste prevention, collection and management practices (UNEP and ISWA, 2024). Although large-scale waste management involves collection, transport, treatment and disposal, effective management also depends on timely information about where waste is accumulating.

A conventional waste bin generally provides no digital indication of its fill condition. Determining whether it requires attention may therefore depend on physical observation. Sensor-based monitoring creates an opportunity to convert a physical condition, such as the distance between a sensor and the accumulated waste, into digital information that can be processed, stored and presented to users. Recent smart-bin literature commonly identifies fill-level sensing, connectivity, sensing and actuation as central capabilities of smart waste containers (Zoumpoulis et al., 2024).

The Internet of Things (IoT) extends this idea by linking physical devices, sensors and software systems. Smart waste research has used microcontrollers and ultrasonic sensors to estimate bin fill levels and transmit the resulting data to software platforms. For example, Pardini et al. (2020) describe an Arduino-based smart waste monitoring framework that uses ultrasonic sensing and a web-server system to display bin status. Such approaches illustrate how low-cost embedded hardware can be used to connect waste containers to information systems.

InteliBin - Smart Waste Management System is an integrated prototype consisting of a physical waste bin, embedded sensing and control hardware, USB serial communication, PC-side software, a backend/API, database and web dashboard. The embedded subsystem uses an Arduino Uno, two HC-SR04 ultrasonic sensors and a servo motor. The Arduino Uno is based on the ATmega328P and provides digital input/output pins and USB connectivity suitable for embedded prototyping (Arduino, n.d.).

The first ultrasonic sensor is used for person detection. Its TRIG and ECHO pins are connected to Arduino digital pins D9 and D10 respectively. A detection threshold of approximately 15 cm is used. When a person approaches within this threshold, the servo motor connected to D6 moves the lid from the closed position of 90 degrees to the open position of 180 degrees. The lid remains open for approximately five seconds and then closes. The system waits for the detected person to leave before accepting another opening event.

The second ultrasonic sensor measures waste fill level. Its TRIG and ECHO pins are connected to D4 and D5. The sensor points downward into the bin and measures the distance to the waste surface. The HC-SR04 operates by emitting an ultrasonic pulse and measuring the echo return time; its datasheet describes non-contact ranging based on the travel time of sound (SparkFun, n.d.). In InteliBin, seven measurements are taken and a median-based approach is used to reduce the effect of noisy values.

The fill-level subsystem was calibrated using an empty-bin distance of 20.9 cm and a full-bin distance of 4.0 cm. The current measured distance is converted to a fill percentage using the calibrated range, and the result is constrained between 0% and 100%. The Arduino then communicates sensor readings and system status to a computer through USB serial communication. PC-side software integrates this information with the backend, database and dashboard.

## 1.2 Problem Statement

Waste-bin monitoring that relies on manual inspection does not inherently provide a digital, continuously available representation of bin fill condition. This can make it difficult to know the state of a bin without physically checking it. At the same time, conventional lids that require manual opening introduce direct contact with the waste-bin surface.

There is therefore a need to investigate a system that can automatically sense bin conditions and user proximity, actuate the lid without direct contact, and communicate useful status information to a software environment. InteliBin addresses this need through an embedded and software prototype that measures waste fill level, detects an approaching person, controls the lid automatically, and communicates sensor and status information for processing, storage and dashboard monitoring.

## 1.3 Research Questions

How can an embedded sensor-based system be integrated with software to provide automated lid operation and waste-bin fill-level monitoring?

How can ultrasonic sensing be used to detect a person approaching a waste bin and automatically control its lid?

How can ultrasonic distance measurements be calibrated and filtered to estimate the percentage fill level of a waste bin?

How can embedded sensor and status information be communicated to a software monitoring environment?

## 1.4 Aim

The aim of this project is to develop an integrated smart waste management prototype that uses embedded sensing, automatic lid control and software monitoring to measure waste-bin fill level, reduce direct contact with the bin lid and present relevant system information through a digital monitoring environment.

## 1.5 Objectives

To implement person detection using an HC-SR04 ultrasonic sensor.

To automatically open and close the waste-bin lid using a servo motor.

To prevent immediate repeated lid triggering while a detected person remains near the bin.

To measure waste fill level using an HC-SR04 ultrasonic sensor.

To calibrate the fill-level sensor and convert measured distance into a 0-100% fill percentage.

To reduce noisy fill-level measurements using a median-based approach over seven readings.

To communicate sensor readings and system status from the Arduino to a PC through USB serial communication.

To integrate the embedded subsystem with a backend/API, database and web dashboard.

To test individual components, subsystem integration and the complete hardware/software system.

## 1.6 Scope

The implemented scope covers a single smart-bin prototype using an Arduino Uno, two HC-SR04 sensors, a servo motor, USB serial communication and integration with a software system comprising a Python PC-side listener, PHP/PDO backend/API, MySQL/MariaDB database named intelibin on port 3307, and HTML/CSS/JavaScript web dashboard. The system performs person detection, automatic lid operation, fill-level measurement, median-based filtering and fill-percentage calculation.

# 2. LITERATURE REVIEW

## 2.1 Introduction

This chapter reviews literature relevant to smart waste management, smart bins, IoT-enabled monitoring, ultrasonic fill-level sensing and automated actuation. The purpose is to establish the technological context of InteliBin and identify the development gap addressed by the prototype. The review does not treat the reference Golf Club Management System thesis as a source of technical content.

## 2.2 Smart Waste Management and IoT

Smart waste management applies digital technologies to improve visibility, control and decision-making across waste-management processes. Kannan et al. (2024) describe Smart Waste Management 4.0 as the application of Industry 4.0 technologies to waste-management activities and emphasize the integration of smart cities, smart enterprises, smart factories and people. At the bin level, sensing technologies can provide information that is otherwise unavailable until a physical inspection occurs.

IoT-based waste systems commonly combine sensing, local processing, communications and a monitoring platform. The sensing layer measures a physical condition; the controller interprets measurements; a communication layer transfers information; and an application layer presents the information. InteliBin follows this general layered concept but, in the implemented prototype, the Arduino communicates with a PC over USB rather than claiming an independent wireless connection.

## 2.3 Smart Bins and Fill-Level Monitoring

A smart bin extends an ordinary waste container by adding sensing, communication, actuation or data-processing capabilities. Zoumpoulis et al. (2024), in a systematic review of smart bins, identify fill-level monitoring and IoT connectivity as common elements and note that sensing and actuation are important smart-bin capabilities. Their review also shows that smart-bin designs vary significantly and that there is no single standardized architecture.

Fill-level information can support more informed collection decisions because it indicates whether a bin is approaching capacity. At larger scale, research on municipal waste collection has investigated how sensor information can be combined with routing and scheduling models. Such optimization is outside the implemented scope of InteliBin, but it provides a broader motivation for obtaining reliable bin-state data.

## 2.4 Ultrasonic Sensing for Waste-Level Detection

Ultrasonic distance sensors are widely used in prototype smart bins because they can measure distance without physical contact. The HC-SR04 contains an ultrasonic transmitter, receiver and control circuitry. The module is triggered by a pulse, emits an ultrasonic burst and returns an echo pulse whose duration is related to the round-trip travel time of sound (SparkFun, n.d.).

For a sensor mounted at the top of a bin and facing downward, the measured distance decreases as waste accumulates. This relationship makes it possible to convert a distance into an estimated fill percentage after calibration. Raw readings can be affected by the shape, orientation and acoustic properties of the waste surface. InteliBin therefore collects seven readings and uses a median-based method, which reduces the influence of isolated unusually high or low values without claiming a numerical accuracy improvement that was not measured.

## 2.5 Automated Lid Systems

Actuation allows a smart bin to respond to sensed conditions rather than only reporting them. In InteliBin, the actuation requirement is automatic lid control. A person-detection sensor measures proximity and the Arduino commands a servo motor when the configured threshold is reached. This creates a touchless interaction path between the user and the bin. The design also includes a state condition that requires the person to leave before the next opening event is accepted, reducing repeated triggering while the same person remains in front of the sensor.

## 2.6 Review of Existing Approaches

Pardini et al. (2020) present an IoT-based smart waste monitoring and control framework using an Arduino microcontroller and ultrasonic sensors to detect garbage levels and send information to a central web-server system. Their work demonstrates the feasibility of combining ultrasonic sensing, microcontroller processing and web visualization.

Zoumpoulis et al. (2024) provide a broader systematic review of smart-bin research and identify recurring capabilities including fill-level monitoring, connectivity, sensing and actuation. Kannan et al. (2024) examine smart waste management from a wider Industry 4.0 perspective, showing that smart waste solutions can be viewed as components of larger digital ecosystems.

| Approach | Relevant Capability | Relation to InteliBin |
| --- | --- | --- |
| Pardini et al. (2020) | Arduino, ultrasonic sensing, web-based monitoring | Similar fill-level sensing and software-monitoring concept; InteliBin also includes automatic lid actuation and uses USB-to-PC communication in the completed prototype. |
| Zoumpoulis et al. (2024) | Systematic review of smart bins; sensing, actuation, connectivity | Provides broader context for the combination of fill sensing and actuation. |
| Kannan et al. (2024) | Smart Waste Management 4.0 framework | Places smart-bin data and monitoring within wider digital waste-management transformation. |
| InteliBin | Person detection, automatic lid, calibrated fill level, filtering, serial-to-software integration | Prototype focuses on integrating touchless actuation and fill monitoring in one embedded/software system. |

## 2.7 Development Gap

Existing literature demonstrates that ultrasonic sensors and microcontrollers can be used for waste-level monitoring, and that smart bins may incorporate sensing, actuation and connectivity. The development focus of InteliBin is not to claim a new sensing principle. Instead, the project integrates several practical functions into a single prototype: person proximity detection, automatic lid operation, prevention of immediate repeated triggering, calibrated fill-level estimation, median-based filtering, USB serial communication and software-side monitoring.

The project therefore addresses an implementation and integration problem: how to construct and verify a working embedded/software chain from physical sensing and actuation through to a digital dashboard using the available hardware and development environment.

## 2.8 Research Methodology

The requirements for InteliBin were informed by background research into smart waste management, system analysis of the required sensing and monitoring functions, and iterative engineering development. Interviews and surveys are not claimed because no evidence of such activities was supplied.

### 2.8.1 Background Research

Background research was used to understand smart-bin architectures, ultrasonic sensing, microcontroller-based control and software monitoring. Technical documentation was also relevant for understanding the Arduino Uno and HC-SR04 operating principles.

### 2.8.2 System Analysis

System analysis decomposed the intended solution into person detection, lid actuation, fill-level sensing, measurement filtering, calibration, serial communication and software monitoring. This decomposition supported the definition of functional requirements and a component-by-component implementation strategy.

### 2.8.3 Iterative Development and Testing

The prototype was developed incrementally. Testing progressed through servo testing, person-sensor testing, automatic lid testing, fill-level sensor testing, calibration, sensor filtering, complete hardware integration, software integration and full-system testing. This sequence reduced the difficulty of diagnosing faults because individual functions could be verified before they were combined.

## 2.9 Summary

The literature establishes that sensing, actuation and connectivity are important elements of smart-bin systems. Ultrasonic sensing provides a practical means of estimating distance to waste, while microcontrollers can coordinate sensing, control and communication. InteliBin builds on these established ideas in a prototype that integrates automatic lid control with calibrated fill-level monitoring and a software monitoring chain.

# 3. REQUIREMENTS ANALYSIS

## 3.1 Development Methodology

An iterative development methodology was used because InteliBin combines hardware, embedded control and software components that must operate together. The system was divided into smaller functions, each of which could be implemented and tested before integration.

The development sequence was: servo testing; person-detection sensor testing; automatic lid testing; fill-level sensor testing; fill-level calibration; median filtering; complete hardware integration; software integration; and full-system testing. This sequence reflects an incremental engineering process in which later stages depend on verified earlier stages.

Figure 1: Iterative development sequence for InteliBin.

As illustrated in Figure 1, InteliBin was developed using an iterative approach in which individual components were implemented, tested, integrated and evaluated before subsequent refinement. Development progressed through nine main stages: servo testing, person-detection sensor testing, automatic lid testing, fill-level sensor testing, fill-level calibration, sensor filtering, complete hardware integration, software integration and full system testing.

## 3.2 Required Resources

| Component | Quantity | Purpose |
| --- | --- | --- |
| Arduino Uno | 1 | Embedded controller for sensors, servo logic and serial communication |
| HC-SR04 ultrasonic sensor | 2 | Person detection and waste fill-level distance measurement |
| Servo motor | 1 | Automatic lid actuation |
| Breadboard | 1 | Prototype circuit interconnection |
| Jumper wires |  | Electrical connections |
| USB cable | 1 | Arduino-PC connection and serial communication |
| Personal computer | 1 | PC-side serial processing and software system |
| Physical waste bin | 1 | Prototype enclosure and waste container |

Software resources verified in the repository include Arduino IDE-compatible C/C++ firmware using the Servo library, Python 3 with pyserial and pymysql for the serial bridge, PHP with PDO for backend/database access, MySQL/MariaDB SQL scripts, and browser-side HTML, CSS and JavaScript for the dashboard.

## 3.3 Functional Requirements

| ID | Functional Requirement |
| --- | --- |
| FR-01 | The system shall detect a person approaching the bin using the person-detection ultrasonic sensor. |
| FR-02 | The system shall use an approximate 15 cm threshold to determine when to initiate automatic lid opening. |
| FR-03 | The system shall command the servo to move the lid to the open position of 180 degrees. |
| FR-04 | The system shall return the lid to the closed position of 90 degrees after approximately five seconds. |
| FR-05 | The system shall wait for the person to leave before accepting another opening trigger. |
| FR-06 | The system shall measure the distance from the fill-level sensor to the waste surface. |
| FR-07 | The system shall obtain seven readings for fill-level processing. |
| FR-08 | The system shall use a median-based method to reduce noisy fill-level measurements. |
| FR-09 | The system shall convert the filtered distance into a fill percentage using the calibrated empty and full distances. |
| FR-10 | The system shall constrain the calculated fill percentage to the range 0-100%. |
| FR-11 | The Arduino shall communicate sensor readings and system status to a PC through USB serial communication. |
| FR-12 | The software system shall process/store relevant information and present monitoring information through a web dashboard. |

## 3.4 Non-Functional Requirements

| ID | Category | Requirement |
| --- | --- | --- |
| NFR-01 | Usability | The monitoring interface should present system information in a clear form. |
| NFR-02 | Reliability | The control logic should behave consistently under valid operating conditions. |
| NFR-03 | Maintainability | Hardware pin assignments, calibration values and software modules should be organized so that changes can be made without redesigning the entire system. |
| NFR-04 | Performance | Person detection, lid control and fill-level processing should respond within the timing implemented by the prototype; no unsupported numerical benchmark is claimed. |
| NFR-05 | Accuracy | Fill-level estimation should use calibration and median-based filtering to reduce measurement instability; no unmeasured accuracy percentage is claimed. |
| NFR-06 | Security | Access to stored or dashboard data is protected by implemented software controls: admin login backed by the admin_users table, password_hash/password_verify password handling, the intelibin_admin PHP session name, session_regenerate_id(true) on login, protected page controllers that call require_admin_login(), logout session destruction, and authenticated API checks on dashboard_state.php and command.php. The repository does not implement role levels or CSRF tokens. |
| NFR-07 | Scalability | The architecture should permit future extension, but the implemented prototype is not claimed to be a deployed multi-bin system. |
| NFR-08 | Availability | The monitoring system depends on a local Apache/PHP server, MySQL/MariaDB on localhost port 3307, the Python serial bridge process, and the Arduino connected over USB serial. The dashboard reports the bin as offline when bin_state.last_seen_at is not refreshed by the bridge heartbeat. No cloud failover or remote redundancy is implemented in the repository. |

# 4. SYSTEM DESIGN

## 4.1 Proposed System

InteliBin was designed as a layered embedded/software system. The physical layer consists of the waste bin, two ultrasonic sensors and a servo. The control layer is the Arduino Uno. Communication from the Arduino to the PC occurs through USB serial communication. The PC-side listener passes or transforms the information for the backend/API, which interacts with the database and dashboard.

[INSERT FIGURE: Physical Bin -> Ultrasonic Sensors + Servo -> Arduino Uno -> USB Serial Communication -> PC/Serial Listener -> Backend/API -> Database -> Web Dashboard.]

Figure 2: InteliBin Overall System Architecture.

## 4.2 Hardware Design

| Device/Signal | Arduino Pin | Function |
| --- | --- | --- |
| Person sensor TRIG | D9 | Triggers proximity measurement |
| Person sensor ECHO | D10 | Receives proximity echo pulse |
| Fill sensor TRIG | D4 | Triggers fill-level distance measurement |
| Fill sensor ECHO | D5 | Receives fill-level echo pulse |
| Servo signal | D6 | Controls lid position |

[INSERT FIGURE: Arduino Uno connected to two HC-SR04 sensors, servo motor, power/ground, breadboard and USB connection. Use the exact D9/D10, D4/D5 and D6 pin assignments.]

Figure 3: InteliBin Hardware Wiring Diagram.

## 4.3 Person Detection and Lid-Control Design

The person-detection sensor measures the distance to an object in front of the bin. When the measured distance is approximately 15 cm or less, the lid-control logic commands the servo to the open position of 180 degrees. The lid remains open for approximately five seconds and then moves to 90 degrees. After closing, the system does not accept another opening event until the person is no longer detected within the triggering condition.

[INSERT FIGURE: Start -> measure person distance -> valid and <=15 cm? -> open servo to 180 degrees -> wait about 5 s -> close to 90 degrees -> check whether person has left -> when clear, return to monitoring.]

Figure 4: Automatic Lid Control Flowchart.

## 4.4 Fill-Level Measurement Design

The fill-level sensor is mounted so that it points downward into the bin. The empty-bin reference distance is 20.9 cm and the full-bin reference distance is 4.0 cm. Seven readings are obtained for a fill-level measurement cycle. The readings are ordered and a median-based value is selected to reduce the influence of isolated noisy measurements.

| Calibration State | Distance |
| --- | --- |
| Empty bin | 20.9 cm |
| Full bin | 4.0 cm |

The filtered current distance is converted to a fill percentage using the following relationship:

Fill Percentage = ((Empty Distance - Current Distance) / (Empty Distance - Full Distance)) x 100

The result is constrained between 0% and 100%. Values below 0% are represented as 0%, while values above 100% are represented as 100%.

[INSERT FIGURE: Take seven distance readings -> validate readings -> sort/select median -> apply 20.9 cm empty and 4.0 cm full calibration -> calculate percentage -> constrain to 0-100% -> output/transmit value.]

Figure 5: Fill-Level Measurement Flowchart.

## 4.5 Software Architecture

The software architecture uses Arduino firmware for sensing and lid control, a Python bridge for serial parsing and MySQL writes, MySQL/MariaDB stored procedures and views for persistence and state, PHP/PDO controllers and API endpoints for authenticated web access, and JavaScript/CSS components for the dashboard user interface.

## 4.6 Data Flow

Data originates from the sensors. The Arduino processes proximity and fill-level measurements and controls the servo. It sends newline-delimited JSON at 9600 baud over USB serial. python/serial_to_mysql.py parses heartbeat, reading, person, lid and ready events, writes readings and state to MySQL using stored procedures, and polls queued commands. PHP controllers read the database, while assets/js/dashboard-ui.js polls api/dashboard_state.php about every 5 seconds. The firmware emits heartbeat messages every 5 seconds, fill readings every 300000 ms, and person checks every 100 ms.

[INSERT FIGURE: Show sensor data entering Arduino; local lid-control output to servo; serial data to PC listener; backend processing; database storage; dashboard presentation.]

Figure 6: InteliBin Data Flow Diagram.

## 4.7 Database Design

A database is part of the completed software architecture, but the actual schema was not supplied with the thesis instructions. The final thesis must include the real entities, attributes, primary keys, foreign keys and relationships. No tables are invented in this draft.

ER diagram source: sql/intelibin_db.sql and sql/intelibin_twoway.sql. Verified entities are admin_users, bins, readings, alerts, bin_state, command_log and person_events. The readings, alerts, bin_state, command_log and person_events tables relate to bins through bin_id; alerts may also reference readings through reading_id.

Figure 7: InteliBin Database Entity Relationship Diagram.

## 4.8 Dashboard Design

The implemented dashboard pages and views include the authenticated dashboard, bins, history, alerts, settings and about pages. Dashboard functions include bin switching, live/offline status, fill percentage display, recent readings, alert summaries, people/event information, lid controls, chart range controls, and data-pipeline status. Supporting source files include components/dashboard/*.php, assets/js/dashboard-ui.js, assets/js/page-transitions.js and assets/css/pages/dashboard.css.

# 5. SYSTEM IMPLEMENTATION

## 5.1 Description of the Developed System

The completed InteliBin prototype integrates the physical waste bin with embedded electronics and a software monitoring system. The embedded portion performs person detection, automatic lid control and fill-level measurement. The software portion receives information from the Arduino through a PC-side serial connection and integrates it with a backend/API, database and dashboard.

## 5.2 Hardware Implementation

### 5.2.1 Arduino Uno

The Arduino Uno serves as the central embedded controller. It receives echo signals from the two HC-SR04 sensors, generates trigger signals, calculates distance values, applies the lid-control logic, calculates the fill level and controls the servo. It also provides serial communication to the connected PC through USB.

### 5.2.2 Person-Detection Sensor

The person-detection HC-SR04 uses D9 as TRIG and D10 as ECHO. Distance is measured repeatedly and compared with the approximate 15 cm detection threshold. A qualifying detection initiates the automatic lid sequence.

### 5.2.3 Fill-Level Sensor

The fill-level HC-SR04 uses D4 as TRIG and D5 as ECHO. It is oriented downward into the bin. The implementation collects seven readings, applies a median-based filtering step and converts the resulting distance using the calibrated empty and full references.

### 5.2.4 Servo and Lid Mechanism

The servo signal is connected to D6. The closed position is 90 degrees and the open position is 180 degrees. During an opening event, the servo moves to 180 degrees, remains open for approximately five seconds, and then returns to 90 degrees.

### 5.2.5 Physical Integration

The source code verifies the electrical contract and firmware calibration: person sensor on D9/D10, fill sensor on D4/D5, servo signal on D6, 15 cm person threshold, closed angle 90, open angle 180, empty fill distance 20.9 cm and full fill distance 4.0 cm. The repository does not contain exact mounting dimensions, mechanical linkage measurements or prototype photographs.

[INSERT FIGURE: Insert a clear photograph of the actual completed bin showing sensor placement, lid/servo mechanism and controller where visible.]

Figure 8: Completed InteliBin Prototype.

## 5.3 Embedded Software Implementation

### 5.3.1 Distance Measurement

Distance measurement is based on the ultrasonic time-of-flight principle. A trigger pulse causes the sensor to emit an ultrasonic burst. The echo duration represents the round-trip travel time. The firmware converts this time into distance. The exact Arduino source code should be included in Appendix A.

### 5.3.2 Person Detection and Leave Logic

The measured person distance is compared with the configured threshold. When the threshold condition is satisfied, the lid is opened. After the timed open period and closing action, the firmware requires the person to leave the triggering region before another opening event is permitted. This state-based behavior prevents continuous reopening caused by the same person remaining in front of the sensor.

### 5.3.3 Median Filtering

For fill-level measurement, seven readings are collected. A median-based process is applied so that the representative value is determined by the middle of the ordered measurements rather than by a single raw reading. This is particularly useful when an isolated measurement is inconsistent with the rest of the sample. No numerical noise-reduction percentage is claimed because such a statistic was not supplied.

### 5.3.4 Calibration and Fill Percentage

Calibration maps physical distance to the application-specific empty and full conditions. InteliBin uses 20.9 cm for empty and 4.0 cm for full. The difference between these values defines the usable fill-depth range. The current filtered distance is normalized within this range and multiplied by 100. The value is then constrained to the valid percentage range.

### 5.3.5 Serial Output

The Arduino provides sensor readings and system status to the PC through USB serial communication at 9600 baud using newline-delimited JSON. Verified message fields include event, personDistance, fillDistance, fillPercentage, lid, detected and error. Example firmware messages are {"event":"heartbeat","personDistance":35.2,"fillDistance":8.4,"fillPercentage":74,"lid":"closed"}, {"event":"reading","personDistance":35.2,"fillDistance":8.4,"fillPercentage":74,"lid":"closed"}, {"event":"person","personDistance":12.0,"detected":true,"fillDistance":8.4,"fillPercentage":74,"lid":"open"} and {"event":"lid","personDistance":12.0,"fillDistance":8.4,"fillPercentage":74,"lid":"closed"}. The bridge also supports legacy DIST:, PERSON:, LID:OPEN and LID:CLOSED messages.

## 5.4 Software Integration

### 5.4.1 PC Serial Listener

The PC-side listener is python/serial_to_mysql.py. It uses pyserial to connect to COM8 at 9600 baud and PyMySQL to connect to MySQL on localhost:3307. It parses JSON messages, validates fillPercentage/fillDistance fields, writes readings with insert_reading, updates person detection with update_person_detection, updates heartbeat/state timestamps in bin_state, polls queued commands every 2 seconds, sends OPEN/CLOSE to the Arduino, and marks commands timed out if they are not confirmed within 15 seconds. It logs malformed JSON, invalid numeric values, database reconnect attempts and serial errors.

### 5.4.2 Backend/API

The backend/API is implemented in PHP using PDO. Verified endpoints include POST api/command.php for authenticated open, close and reset commands using the issue_command stored procedure; GET api/dashboard_state.php for authenticated dashboard state JSON; GET api/fetch_latest.php for the latest reading by bin_id; and POST api/simulate_reading.php for inserting simulated readings through insert_reading. Page controllers are under pages/controllers and shared UI/layout helpers are under includes and components.

### 5.4.3 Database

The database is MySQL/MariaDB. The main schema in sql/intelibin_db.sql defines admin_users, bins, readings and alerts, with latest_reading and daily_summary views and an insert_reading stored procedure. sql/intelibin_twoway.sql adds bin_state, command_log and person_events, refreshes latest_reading with live state fields, and adds issue_command, confirm_command and update_person_detection procedures. No automated purge or retention policy is implemented in the SQL files.

### 5.4.4 Web Dashboard

Dashboard source files are verified in pages/controllers/dashboard.php, components/dashboard/*.php, assets/js/dashboard-ui.js, assets/js/dashboard.js and assets/css/pages/dashboard.css. The interface provides live/offline status, fill display, charting controls, recent readings and alerts, person/lid state, remote lid commands and navigation to bins, history, alerts, settings and about pages.

[INSERT FIGURE: Insert an actual screenshot of the implemented dashboard.]

Figure 9: Web Dashboard.

[INSERT FIGURE: Insert only if this screen or dashboard component actually exists.]

Figure 10: Fill-Level Monitoring Interface.

[INSERT FIGURE: Insert only if this screen or dashboard component actually exists.]

Figure 11: Sensor/Lid Status Interface.

## 5.5 Complete System Integration

Hardware/software integration was completed successfully. The final system links sensor measurements and embedded control with PC-side communication and the software monitoring environment. This result verifies that the project moved beyond isolated hardware demonstrations to an integrated prototype.

# 6. TESTING AND VERIFICATION

## 6.1 Testing Methodology

Testing was organized at three levels: component testing, integration testing and complete-system testing. Component testing verified individual hardware and embedded functions. Integration testing verified communication and interaction between components. System testing considered the completed InteliBin chain. This structure reflects the actual incremental development process and makes it possible to relate test evidence directly to requirements.

## 6.2 Component Testing

| ID | Test | Input/Condition | Expected Result | Actual Result | Status |
| --- | --- | --- | --- | --- | --- |
| CT-01 | Servo movement | Command servo to closed/open positions | Servo moves between 90 and 180 degrees | Servo movement worked correctly | Pass |
| CT-02 | Person sensor distance | Place an object/person at different observable distances | Sensor produces distance readings | Person sensor detected distances correctly | Pass |
| CT-03 | Person threshold | Approach within approximately 15 cm | Detection condition becomes true | Detection triggered automatic lid opening | Pass |
| CT-04 | Lid timing/positions | Trigger an opening event | Open to 180 degrees, remain open ~5 s, close to 90 degrees | Observed as expected | Pass |
| CT-05 | Leave-before-retrigger logic | Remain within detection area after an opening event | System does not immediately accept another trigger until person leaves | System waited for the person to leave | Pass |
| CT-06 | Fill sensor distance | Place material/target beneath fill sensor | Sensor produces distance readings | Fill-level sensor produced distance readings | Pass |
| CT-07 | Calibration | Measure empty and full reference positions | References established | 20.9 cm empty; 4.0 cm full | Pass |
| CT-08 | Median filtering | Process seven fill-level readings | Median-based representative value used | Seven-reading median filtering implemented | Pass |

## 6.3 Integration Testing

| ID | Integration | Condition | Expected Result | Actual Result | Status |
| --- | --- | --- | --- | --- | --- |
| IT-01 | Person sensor + servo | Approach within threshold | Sensor event causes lid opening | Automatic lid opening succeeded | Pass |
| IT-02 | Fill sensor + Arduino | Obtain fill readings | Arduino receives/processes distance data | Complete hardware test succeeded | Pass |
| IT-03 | Arduino + PC | Connect Arduino by USB and run integration | PC receives serial sensor/status information | Hardware/software integration completed successfully | Pass |
| IT-04 | PC + backend | Transmit/process received data | Data reaches backend as implemented | Evidence verified from source: python/serial_to_mysql.py parses Arduino JSON and supported legacy serial lines, writes readings through insert_reading, updates person detection through update_person_detection, refreshes bin_state timestamps, and handles queued commands from the backend. | Pass at source-code integration level: the PC bridge-to-backend processing path is implemented. The repository does not include a separate formal test log for this case. |
| IT-05 | Backend + database | Process data requiring persistence | Relevant data stored/retrieved | Evidence verified from source: sql/intelibin_db.sql stores readings and alerts, sql/intelibin_twoway.sql stores live state, commands and person events, and the Python bridge calls these stored procedures when serial data arrives. | Pass at source-code integration level: backend and bridge writes persist to the database schema. The repository does not include a separate measured database test report. |
| IT-06 | Backend + dashboard | Open dashboard with system operating | Monitoring information displayed | Evidence verified from source: pages/controllers/dashboard.php builds the dashboard view model from the database, api/dashboard_state.php returns authenticated JSON, and assets/js/dashboard-ui.js polls that endpoint and updates the dashboard display. | Pass at source-code integration level: the backend-to-dashboard data path is implemented. The repository does not include a separate browser test report. |

## 6.4 System Testing

The complete hardware test succeeded, and hardware/software integration was subsequently completed successfully. This confirms that the core embedded functions operated together and that the embedded subsystem was connected to the software system. A final end-to-end dashboard test record should be inserted from the actual implementation evidence.

| ID | Requirement | Test Description | Expected Result | Actual Result | Status |
| --- | --- | --- | --- | --- | --- |
| ST-01 | Integrated hardware | Operate person detection, lid and fill sensing together | All hardware functions operate without preventing each other | Complete hardware test succeeded | Pass |
| ST-02 | Hardware/software integration | Run hardware while connected to software system | Sensor/status information is communicated into software chain | Integration completed successfully | Pass |
| ST-03 | End-to-end dashboard | Operate complete system and observe dashboard | Actual implemented dashboard reflects received/stored information | Evidence verified from source: the full implemented path is Arduino JSON over USB serial to python/serial_to_mysql.py, then MySQL stored procedures and views, then PHP dashboard controllers and JavaScript dashboard polling. | Pass at source-code integration level. The repository does not contain a formal lab run sheet or screenshot evidence for this system test. |

## 6.5 Analysis of Test Results

The available test evidence demonstrates that the principal hardware requirements were achieved. The servo moved correctly between the configured positions. The person sensor produced usable distance readings and successfully triggered the automatic lid behavior. The lid opened to 180 degrees, remained open for approximately five seconds and returned to 90 degrees. The retrigger-prevention logic also worked by waiting for the person to leave.

The fill-level subsystem produced distance readings and was calibrated using the actual prototype's empty and full distances. Seven-reading median filtering was implemented. These results establish that the fill-level pipeline progressed from raw sensing through filtering and calibration.

The complete hardware test succeeded and hardware/software integration was completed successfully. However, this thesis does not invent numerical accuracy, latency, reliability or network-performance results. Such quantitative evaluation can only be added if actual measurements are provided.

## 6.6 Objective-Implementation-Verification Mapping

| Objective | Implemented Feature | Verification | Result |
| --- | --- | --- | --- |
| Person detection | HC-SR04 on D9/D10, ~15 cm threshold | CT-02, CT-03 | Implemented and passed |
| Automatic lid | Servo on D6; 180 open/90 closed; ~5 s | CT-04, IT-01 | Implemented and passed |
| Prevent repeated trigger | Person-leave logic | CT-05 | Implemented and passed |
| Fill-level measurement | HC-SR04 on D4/D5 | CT-06 | Implemented and passed |
| Calibration and percentage | 20.9 cm empty; 4.0 cm full; 0-100% | CT-07 | Implemented; calibration established |
| Noise reduction | Median of seven readings | CT-08 | Implemented |
| Arduino-PC communication | USB serial connection | IT-03 | Integration completed successfully |
| Backend/database/dashboard integration | Software chain | IT-04 to IT-06, ST-03 | Implementation evidence available in the repository includes firmware constants and serial protocol examples, the Python serial bridge, MySQL schema/procedures/views, PHP API/controller files, and dashboard JavaScript/CSS/components. No separate benchmark dataset, field-test log or screenshot archive is present in the repository. |

# 7. CONCLUSION

## 7.1 Results and Achievements

The InteliBin project achieved the integration of sensing, actuation and software communication in a smart waste-bin prototype. Person proximity was detected using an ultrasonic sensor and used to trigger automatic lid opening. The servo successfully operated between the configured 90-degree closed and 180-degree open positions, with an approximately five-second open period. The system also implemented logic requiring the person to leave before another opening event.

Waste fill-level measurement was implemented using a second ultrasonic sensor. The system established empty and full calibration distances of 20.9 cm and 4.0 cm respectively, collected seven readings and applied median-based filtering before calculating a constrained 0-100% fill value.

The embedded hardware was integrated successfully, and hardware/software integration was subsequently completed. The resulting architecture connects the physical bin to an Arduino, PC-side serial communication, backend/API, database and web dashboard. The exact software implementation details remain to be inserted where marked.

## 7.2 Limitations and Problems Faced

Only limitations supported by the actual project experience should be presented as confirmed. The following possible limitations require confirmation before submission:

The prototype depends on a PC/USB connection for communication with the software system; the implemented path is Arduino USB serial to a Python bridge running on the PC.

Ultrasonic readings can be influenced by measurement variation; the firmware uses median filtering of seven HC-SR04 fill readings with at least five valid samples. The repository does not contain a separate waste-surface experiment.

The system is configured at prototype scale in source code with BIN_ID = 1 and SERIAL_PORT = COM8. The repository does not include large-scale deployment evidence.

The repository documents Arduino USB serial communication and 5V/GND wiring for the sensors and servo. It does not specify a separate external power supply arrangement.

Implementation issues reflected in the source include COM port configuration in the Python bridge, MySQL port 3307 configuration, heartbeat-based offline detection through bin_state.last_seen_at, median filtering of HC-SR04 fill readings in firmware, and command timeout handling in the bridge. The repository does not contain a separate wiring or mechanical troubleshooting log.

## 7.3 Future Work and Recommendations

The following items are potential future extensions rather than implemented InteliBin features. They should remain clearly separated from the completed system:

Standalone operation without dependence on a PC, if the current USB/PC dependency is confirmed.

Use of an IoT-capable controller such as an ESP32 for direct wireless communication.

Wireless or cloud-based monitoring.

Support for multiple bins from a single dashboard.

Notifications when configured fill thresholds are reached.

GPS/location information for distributed bins.

Battery or solar power for more independent deployment.

Improved sensing or sensor fusion for difficult waste surfaces.

Collection analytics and, at larger scale, predictive or route-optimization functions.

A mobile application if required by future users.

Larger-scale field deployment and longer-duration testing.

## 7.4 Conclusion

InteliBin demonstrates how embedded hardware and software can be combined to create a smart waste-bin prototype with both automatic physical interaction and digital monitoring capabilities. The project verified person detection, automatic lid control, calibrated fill-level sensing, median-based filtering, complete hardware operation and hardware/software integration. The work provides a foundation for future extensions toward more independent, networked and larger-scale smart waste monitoring while keeping the claims of the present thesis limited to functionality that was actually implemented and tested.

# 8. APPENDICES

## Appendix A: Arduino Source Code

[INSERT ACTUAL ARDUINO SOURCE CODE. Do not include passwords, API keys or credentials.]

## Appendix B: Hardware Wiring and Pin Configuration

| Component | Pin/Connection |
| --- | --- |
| Person HC-SR04 TRIG | Arduino D9 |
| Person HC-SR04 ECHO | Arduino D10 |
| Fill HC-SR04 TRIG | Arduino D4 |
| Fill HC-SR04 ECHO | Arduino D5 |
| Servo signal | Arduino D6 |
| Arduino-PC | USB |

## Appendix C: Software Installation and Deployment

Verified deployment steps are: install and run Apache/PHP and MySQL/MariaDB through the local XAMPP-style environment; ensure MySQL listens on port 3307; import sql/intelibin_db.sql and then sql/intelibin_twoway.sql into the intelibin database; confirm includes/db.php points to localhost, port 3307, database intelibin, user root and blank password unless changed locally; install Python dependencies with pip install pyserial pymysql; upload arduino/intelibin/intelibin.ino to an Arduino Uno with the documented D9/D10, D4/D5 and D6 wiring; set SERIAL_PORT and BIN_ID in python/serial_to_mysql.py; run python python/serial_to_mysql.py; open /intelibin/login.php and sign in with an admin user such as the seeded admin/admin1234 account if the seed SQL has been used.

## Appendix D: Database Schema

Schema summary from the SQL files: admin_users(id, username, password_hash, display_name, is_active, created_at, last_login_at); bins(id, name, location, capacity_liters, description, active, created_at, updated_at); readings(id, bin_id, fill_percent, distance_cm, status, recorded_at); alerts(id, bin_id, reading_id, fill_percent, triggered_at, resolved, resolved_at); bin_state(bin_id, lid_status, command, command_issued_at, command_issued_by, command_override, command_log_id, person_detected, last_seen_at, last_updated); command_log(id, bin_id, command, issued_by, override_full, status, requested_at, completed_at); person_events(id, bin_id, person_detected, distance_cm, detected_at). Views: latest_reading and daily_summary. Procedures: insert_reading, issue_command, confirm_command and update_person_detection.

## Appendix E: API and Serial Communication Examples

Serial/API examples verified from source: Arduino baud rate 9600; example serial JSON {"event":"reading","personDistance":35.2,"fillDistance":8.4,"fillPercentage":74,"lid":"closed"}; PC-to-Arduino commands OPEN and CLOSE; command API request POST api/command.php with {"bin_id":1,"command":"open","issued_by":"Operator","override":0}; dashboard state request GET api/dashboard_state.php?bin_id=1; latest reading request GET api/fetch_latest.php?bin_id=1; simulated reading request POST api/simulate_reading.php with bin_id, fill_percent and optional distance_cm.

## Appendix F: Selected Backend Code

Selected backend implementation points verified in code: api/command.php calls CALL issue_command(:bin_id, :command, :issued_by, :override) after authentication; api/dashboard_state.php returns ib_state_json(ib_vm($vm ?? [])) after authentication; python/serial_to_mysql.py calls insert_reading, update_person_detection and confirm_command; includes/auth.php verifies admin passwords with password_verify and regenerates the PHP session ID on login; sql/intelibin_db.sql creates alerts automatically when insert_reading receives a fill percentage of 80 or higher.

## Appendix G: Additional Test Evidence

[INSERT ACTUAL photographs, serial-monitor output, screenshots, logs or recorded test observations.]

# 9. REFERENCES

Arduino. (n.d.). Arduino UNO R3. Arduino Documentation. https://docs.arduino.cc/hardware/uno-rev3

Kannan, D., Khademolqorani, S., Janatyan, N., & Alavi, S. (2024). Smart waste management 4.0: The transition from a systematic review to an integrated framework. Waste Management, 174, 1-14. https://doi.org/10.1016/j.wasman.2023.08.041

Pardini, K., Rodrigues, J. J. P. C., Kozlov, S. A., Kumar, N., & Furtado, V. (2020). IoT-based framework for smart waste monitoring and control system: A case study for smart cities. Engineering Proceedings, 2(1), 90.

SparkFun Electronics. (n.d.). HC-SR04 ultrasonic ranging module datasheet. https://cdn.sparkfun.com/assets/a/8/9/c/a/HC-SR04-33_Datasheet.pdf

United Nations Environment Programme (UNEP) & International Solid Waste Association (ISWA). (2024). Global Waste Management Outlook 2024: Beyond an Age of Waste - Turning Rubbish into a Resource. UNEP. https://doi.org/10.59117/20.500.11822/44939

Zoumpoulis, P., Konstantinidis, F. K., Tsimiklis, G., & Amditis, A. (2024). Smart bins for enhanced resource recovery and sustainable urban waste practices in smart cities: A systematic literature review. Cities, 152, 105150.

Finalization note: repository-verifiable implementation placeholders have been completed from the source code. Items not present in the repository, such as personal dedication text, acknowledgements, prototype photographs, exact mounting dimensions and formal external test logs, are explicitly identified as not specified rather than invented.