THE UNIVERSITY OF ZAMBIA

SCHOOL OF NATURAL SCIENCES

DEPARTMENT OF COMPUTER SCIENCE

INTELIBIN - SMART WASTE MANAGEMENT SYSTEM

BY

BAUTIS CHILESHE

2022036565

CSC 4004 FINAL YEAR COURSE PROJECT

A THESIS SUBMITTED IN PARTIAL FULFILMENT OF THE REQUIREMENTS FOR THE AWARD OF A BACHELOR'S DEGREE IN COMPUTER SCIENCE

SUPERVISOR: Mrs KABEMBA

2026

# ABSTRACT

Waste-bin monitoring can depend on manual inspection, while conventional bin lids may require direct physical contact. This project presents InteliBin, a Smart Waste Management System that integrates embedded sensing, automatic lid control, serial communication, data processing, storage and a web-based monitoring interface. The prototype uses an Arduino Uno, two HC-SR04 ultrasonic sensors and a servo motor. One ultrasonic sensor detects a person approaching the bin at an approximate threshold of 15 cm. When a person is detected, the servo opens the lid to 180 degrees, keeps it open for approximately five seconds and returns it to the closed position of 90 degrees. The control logic waits for the person to leave before permitting another opening event.

A second HC-SR04 sensor is directed into the bin to estimate waste fill level. Seven distance readings are collected and a median-based approach is used to reduce the influence of noisy measurements. Calibration was established using 20.9 cm as the empty-bin distance and 4.0 cm as the full-bin distance. The measured distance is converted into a percentage between 0% and 100%. Sensor readings and system status are communicated from the Arduino to a personal computer through USB serial communication, after which PC-side software integrates the embedded subsystem with a backend/API, database and web dashboard.

Component, integration and complete-system tests confirmed successful servo movement, person-distance detection, automatic lid operation, fill-level distance measurement, median filtering and hardware/software integration. The project demonstrates a practical integration of embedded hardware and software for smart-bin monitoring. Details that were not supplied for this thesis draft, including the exact software stack, database schema and dashboard screens, are explicitly marked [INFORMATION REQUIRED] rather than inferred.

# DECLARATION

I, BAUTIS CHILESHE, declare that the work presented in this thesis, entitled InteliBin - Smart Waste Management System, is my own work except where sources have been acknowledged. This thesis has not been submitted for the award of a degree at another institution.

Author Signature	: ____________________________
Date			: ____________________________

Supervisor		: ____________________________
Supervisor Signature	: ____________________________

# DEDICATION

[INFORMATION REQUIRED: Insert the student's dedication.]

# ACKNOWLEDGEMENTS

[INFORMATION REQUIRED: Insert acknowledgements to the supervisor, family, colleagues, institution and any other contributors who should be acknowledged.]

Table of Contents

ABSTRACT	i

DECLARATION	ii

DEDICATION	iii

ACKNOWLEDGEMENTS	iv

LIST OF FIGURES	xi

LIST OF TABLES	xii

1. INTRODUCTION	1

1.1 Background	1

1.2 Problem Statement	2

1.3 Research Questions	3

1.4 Aim	3

1.5 Objectives	3

1.6 Scope	4

2. LITERATURE REVIEW	5

2.1 Introduction	5

2.2 Smart Waste Management and IoT	5

2.3 Smart Bins and Fill-Level Monitoring	5

2.4 Ultrasonic Sensing for Waste-Level Detection	6

2.5 Automated Lid Systems	6

2.6 Review of Existing Approaches	6

2.7 Development Gap	7

2.8 Research Methodology	7

2.8.1 Background Research	8

2.8.2 System Analysis	8

2.8.3 Iterative Development and Testing	8

2.9 Summary	8

3. REQUIREMENTS ANALYSIS	9

3.1 Development Methodology	9

3.2 Required Resources	9

3.3 Functional Requirements	10

3.4 Non-Functional Requirements	10

4. SYSTEM DESIGN	11

4.1 Proposed System	11

4.2 Hardware Design	11

4.3 Person Detection and Lid-Control Design	11

4.4 Fill-Level Measurement Design	12

4.5 Software Architecture	12

4.6 Data Flow	13

4.7 Database Design	13

4.8 Dashboard Design	13

5. SYSTEM IMPLEMENTATION	14

5.1 Description of the Developed System	14

5.2 Hardware Implementation	14

5.2.1 Arduino Uno	14

5.2.2 Person-Detection Sensor	14

5.2.3 Fill-Level Sensor	14

5.2.4 Servo and Lid Mechanism	14

5.2.5 Physical Integration	15

5.3 Embedded Software Implementation	15

5.3.1 Distance Measurement	15

5.3.2 Person Detection and Leave Logic	15

5.3.3 Median Filtering	15

5.3.4 Calibration and Fill Percentage	15

5.3.5 Serial Output	16

5.4 Software Integration	16

5.4.1 PC Serial Listener	16

5.4.2 Backend/API	16

5.4.3 Database	16

5.4.4 Web Dashboard	16

5.5 Complete System Integration	17

6. TESTING AND VERIFICATION	18

6.1 Testing Methodology	18

6.2 Component Testing	18

6.3 Integration Testing	18

6.4 System Testing	19

6.5 Analysis of Test Results	19

6.6 Objective-Implementation-Verification Mapping	20

7. CONCLUSION	21

7.1 Results and Achievements	21

7.2 Limitations and Problems Faced	21

7.3 Future Work and Recommendations	22

7.4 Conclusion	22

8. APPENDICES	23

Appendix A: Arduino Source Code	23

Appendix B: Hardware Wiring and Pin Configuration	23

Appendix C: Software Installation and Deployment	23

Appendix D: Database Schema	23

Appendix E: API and Serial Communication Examples	23

Appendix F: Selected Backend Code	23

Appendix G: Additional Test Evidence	23

9. REFERENCES	25

# LIST OF FIGURES

Figure 1: InteliBin Overall System Architecture

Figure 2: InteliBin Hardware Wiring Diagram

Figure 3: Automatic Lid Control Flowchart

Figure 4: Fill-Level Measurement Flowchart

Figure 5: InteliBin Data Flow Diagram

Figure 6: Database Entity Relationship Diagram

Figure 7: Completed InteliBin Prototype

Figure 8: Web Dashboard

Figure 9: Fill-Level Monitoring Interface

Figure 10: Sensor/Lid Status Interface

# LIST OF TABLES

Table 1: Comparison of Selected Smart Waste Approaches

Table 2: InteliBin Hardware Components

Table 3: Arduino Pin Configuration

Table 4: Functional Requirements

Table 5: Non-Functional Requirements

Table 6: Fill-Level Calibration Parameters

Table 7: Component Test Cases

Table 8: Integration Test Cases

Table 9: System Test Cases

Table 10: Objective-Implementation-Verification Matrix

# 1. INTRODUCTION

## 1.1 Background

Waste management is an important service for communities, institutions and cities. The United Nations Environment Programme (UNEP) reports that global municipal solid waste generation is expected to increase substantially toward 2050, reinforcing the need for improved waste prevention, collection and management practices (UNEP and ISWA, 2024). Although large-scale waste management involves collection, transport, treatment and disposal, effective management also depends on timely information about where waste is accumulating.

A conventional waste bin generally provides no digital indication of its fill condition. Determining whether it requires attention may therefore depend on physical observation. Sensor-based monitoring creates an opportunity to convert a physical condition, such as the distance between a sensor and the accumulated waste, into digital information that can be processed, stored and presented to users. Recent smart-bin literature commonly identifies fill-level sensing, connectivity, sensing and actuation as central capabilities of smart waste containers (Zoumpoulis et al., 2024).

The Internet of Things (IoT) extends this idea by linking physical devices, sensors and software systems. Smart waste research has used microcontrollers and ultrasonic sensors to estimate bin fill levels and transmit the resulting data to software platforms. For example, Pardini et al. (2020) describe an Arduino-based smart waste monitoring framework that uses ultrasonic sensing and a web-server system to display bin status. Such approaches illustrate how low-cost embedded hardware can be used to connect waste containers to information systems.

InteliBin - Smart Waste Management System is an integrated prototype consisting of a physical waste bin, embedded sensing and control hardware, USB serial communication, PC-side software, a backend/API, database and web dashboard. The embedded subsystem uses an Arduino Uno, two HC-SR04 ultrasonic sensors and a servo motor. The Arduino Uno is based on the ATmega328P and provides digital input/output pins and USB connectivity suitable for embedded prototyping (Arduino, n.d.).

The first ultrasonic sensor is used for person detection. Its TRIG and ECHO pins are connected to Arduino digital pins D9 and D10 respectively. A detection threshold of approximately 15 cm is used. When a person approaches within this threshold, the servo motor connected to D6 moves the lid from the closed position of 90 degrees to the open position of 180 degrees. The lid remains open for approximately five seconds and then closes. The system waits for the detected person to leave before accepting another opening event.

The second ultrasonic sensor measures waste fill level. Its TRIG and ECHO pins are connected to D4 and D5. The sensor points downward into the bin and measures the distance to the waste surface. The HC-SR04 operates by emitting an ultrasonic pulse and measuring the echo return time; its datasheet describes non-contact ranging based on the travel time of sound (SparkFun, n.d.). In InteliBin, seven measurements are taken and a median-based approach is used to reduce the effect of noisy values.

The fill-level subsystem was calibrated using an empty-bin distance of 20.9 cm and a full-bin distance of 4.0 cm. The current measured distance is converted to a fill percentage using the calibrated range, and the result is constrained between 0% and 100%. The Arduino then communicates sensor readings and system status to a computer through USB serial communication. PC-side software integrates this information with the backend, database and dashboard.

## 1.2 Problem Statement

Waste-bin monitoring that relies on manual inspection does not inherently provide a digital, continuously available representation of bin fill condition. This can make it difficult to know the state of a bin without physically checking it. At the same time, conventional lids that require manual opening introduce direct contact with the waste-bin surface.

There is therefore a need to investigate a system that can automatically sense bin conditions and user proximity, actuate the lid without direct contact, and communicate useful status information to a software environment. InteliBin addresses this need through an embedded and software prototype that measures waste fill level, detects an approaching person, controls the lid automatically, and communicates sensor and status information for processing, storage and dashboard monitoring.

## 1.3 Research Questions

How can an embedded sensor-based system be integrated with software to provide automated lid operation and waste-bin fill-level monitoring?

How can ultrasonic sensing be used to detect a person approaching a waste bin and automatically control its lid?

How can ultrasonic distance measurements be calibrated and filtered to estimate the percentage fill level of a waste bin?

How can embedded sensor and status information be communicated to a software monitoring environment?

## 1.4 Aim

The aim of this project is to develop an integrated smart waste management prototype that uses embedded sensing, automatic lid control and software monitoring to measure waste-bin fill level, reduce direct contact with the bin lid and present relevant system information through a digital monitoring environment.

## 1.5 Objectives

To implement person detection using an HC-SR04 ultrasonic sensor.

To automatically open and close the waste-bin lid using a servo motor.

To prevent immediate repeated lid triggering while a detected person remains near the bin.

To measure waste fill level using an HC-SR04 ultrasonic sensor.

To calibrate the fill-level sensor and convert measured distance into a 0-100% fill percentage.

To reduce noisy fill-level measurements using a median-based approach over seven readings.

To communicate sensor readings and system status from the Arduino to a PC through USB serial communication.

To integrate the embedded subsystem with a backend/API, database and web dashboard.

To test individual components, subsystem integration and the complete hardware/software system.

## 1.6 Scope

The implemented scope covers a single smart-bin prototype using an Arduino Uno, two HC-SR04 sensors, a servo motor, USB serial communication and integration with a software system comprising a PC-side listener, backend/API, database and web dashboard. The system performs person detection, automatic lid operation, fill-level measurement, median-based filtering and fill-percentage calculation. The exact technologies used for the PC listener, backend, database and dashboard are [INFORMATION REQUIRED]. Features such as wireless networking, GPS, mobile applications, cloud deployment, route optimization and multi-bin fleet management are not claimed as implemented.

# 2. LITERATURE REVIEW

## 2.1 Introduction

This chapter reviews literature relevant to smart waste management, smart bins, IoT-enabled monitoring, ultrasonic fill-level sensing and automated actuation. The purpose is to establish the technological context of InteliBin and identify the development gap addressed by the prototype. The review does not treat the reference Golf Club Management System thesis as a source of technical content.

## 2.2 Smart Waste Management and IoT

Smart waste management applies digital technologies to improve visibility, control and decision-making across waste-management processes. Kannan et al. (2024) describe Smart Waste Management 4.0 as the application of Industry 4.0 technologies to waste-management activities and emphasize the integration of smart cities, smart enterprises, smart factories and people. At the bin level, sensing technologies can provide information that is otherwise unavailable until a physical inspection occurs.

IoT-based waste systems commonly combine sensing, local processing, communications and a monitoring platform. The sensing layer measures a physical condition; the controller interprets measurements; a communication layer transfers information; and an application layer presents the information. InteliBin follows this general layered concept but, in the implemented prototype, the Arduino communicates with a PC over USB rather than claiming an independent wireless connection.

## 2.3 Smart Bins and Fill-Level Monitoring

A smart bin extends an ordinary waste container by adding sensing, communication, actuation or data-processing capabilities. Zoumpoulis et al. (2024), in a systematic review of smart bins, identify fill-level monitoring and IoT connectivity as common elements and note that sensing and actuation are important smart-bin capabilities. Their review also shows that smart-bin designs vary significantly and that there is no single standardized architecture.

Fill-level information can support more informed collection decisions because it indicates whether a bin is approaching capacity. At larger scale, research on municipal waste collection has investigated how sensor information can be combined with routing and scheduling models. Such optimization is outside the implemented scope of InteliBin, but it provides a broader motivation for obtaining reliable bin-state data.

## 2.4 Ultrasonic Sensing for Waste-Level Detection

Ultrasonic distance sensors are widely used in prototype smart bins because they can measure distance without physical contact. The HC-SR04 contains an ultrasonic transmitter, receiver and control circuitry. The module is triggered by a pulse, emits an ultrasonic burst and returns an echo pulse whose duration is related to the round-trip travel time of sound (SparkFun, n.d.).

For a sensor mounted at the top of a bin and facing downward, the measured distance decreases as waste accumulates. This relationship makes it possible to convert a distance into an estimated fill percentage after calibration. Raw readings can be affected by the shape, orientation and acoustic properties of the waste surface. InteliBin therefore collects seven readings and uses a median-based method, which reduces the influence of isolated unusually high or low values without claiming a numerical accuracy improvement that was not measured.

## 2.5 Automated Lid Systems

Actuation allows a smart bin to respond to sensed conditions rather than only reporting them. In InteliBin, the actuation requirement is automatic lid control. A person-detection sensor measures proximity and the Arduino commands a servo motor when the configured threshold is reached. This creates a touchless interaction path between the user and the bin. The design also includes a state condition that requires the person to leave before the next opening event is accepted, reducing repeated triggering while the same person remains in front of the sensor.

## 2.6 Review of Existing Approaches

Pardini et al. (2020) present an IoT-based smart waste monitoring and control framework using an Arduino microcontroller and ultrasonic sensors to detect garbage levels and send information to a central web-server system. Their work demonstrates the feasibility of combining ultrasonic sensing, microcontroller processing and web visualization.

Zoumpoulis et al. (2024) provide a broader systematic review of smart-bin research and identify recurring capabilities including fill-level monitoring, connectivity, sensing and actuation. Kannan et al. (2024) examine smart waste management from a wider Industry 4.0 perspective, showing that smart waste solutions can be viewed as components of larger digital ecosystems.

| Approach | Relevant Capability | Relation to InteliBin |
| --- | --- | --- |
| Pardini et al. (2020) | Arduino, ultrasonic sensing, web-based monitoring | Similar fill-level sensing and software-monitoring concept; InteliBin also includes automatic lid actuation and uses USB-to-PC communication in the completed prototype. |
| Zoumpoulis et al. (2024) | Systematic review of smart bins; sensing, actuation, connectivity | Provides broader context for the combination of fill sensing and actuation. |
| Kannan et al. (2024) | Smart Waste Management 4.0 framework | Places smart-bin data and monitoring within wider digital waste-management transformation. |
| InteliBin | Person detection, automatic lid, calibrated fill level, filtering, serial-to-software integration | Prototype focuses on integrating touchless actuation and fill monitoring in one embedded/software system. |

## 2.7 Development Gap

Existing literature demonstrates that ultrasonic sensors and microcontrollers can be used for waste-level monitoring, and that smart bins may incorporate sensing, actuation and connectivity. The development focus of InteliBin is not to claim a new sensing principle. Instead, the project integrates several practical functions into a single prototype: person proximity detection, automatic lid operation, prevention of immediate repeated triggering, calibrated fill-level estimation, median-based filtering, USB serial communication and software-side monitoring.

The project therefore addresses an implementation and integration problem: how to construct and verify a working embedded/software chain from physical sensing and actuation through to a digital dashboard using the available hardware and development environment.

## 2.8 Research Methodology

The requirements for InteliBin were informed by background research into smart waste management, system analysis of the required sensing and monitoring functions, and iterative engineering development. Interviews and surveys are not claimed because no evidence of such activities was supplied.

### 2.8.1 Background Research

Background research was used to understand smart-bin architectures, ultrasonic sensing, microcontroller-based control and software monitoring. Technical documentation was also relevant for understanding the Arduino Uno and HC-SR04 operating principles.

### 2.8.2 System Analysis

System analysis decomposed the intended solution into person detection, lid actuation, fill-level sensing, measurement filtering, calibration, serial communication and software monitoring. This decomposition supported the definition of functional requirements and a component-by-component implementation strategy.

### 2.8.3 Iterative Development and Testing

The prototype was developed incrementally. Testing progressed through servo testing, person-sensor testing, automatic lid testing, fill-level sensor testing, calibration, sensor filtering, complete hardware integration, software integration and full-system testing. This sequence reduced the difficulty of diagnosing faults because individual functions could be verified before they were combined.

## 2.9 Summary

The literature establishes that sensing, actuation and connectivity are important elements of smart-bin systems. Ultrasonic sensing provides a practical means of estimating distance to waste, while microcontrollers can coordinate sensing, control and communication. InteliBin builds on these established ideas in a prototype that integrates automatic lid control with calibrated fill-level monitoring and a software monitoring chain.

# 3. REQUIREMENTS ANALYSIS

## 3.1 Development Methodology

An iterative development methodology was used because InteliBin combines hardware, embedded control and software components that must operate together. The system was divided into smaller functions, each of which could be implemented and tested before integration.

The development sequence was: servo testing; person-detection sensor testing; automatic lid testing; fill-level sensor testing; fill-level calibration; median filtering; complete hardware integration; software integration; and full-system testing. This sequence reflects an incremental engineering process in which later stages depend on verified earlier stages.

Figure 1: Iterative development sequence for InteliBin.

As illustrated in Figure 1, InteliBin was developed using an iterative approach in which individual components were implemented, tested, integrated and evaluated before subsequent refinement. Development progressed through nine main stages: servo testing, person-detection sensor testing, automatic lid testing, fill-level sensor testing, fill-level calibration, sensor filtering, complete hardware integration, software integration and full system testing.

## 3.2 Required Resources

| Component | Quantity | Purpose |
| --- | --- | --- |
| Arduino Uno | 1 | Embedded controller for sensors, servo logic and serial communication |
| HC-SR04 ultrasonic sensor | 2 | Person detection and waste fill-level distance measurement |
| Servo motor | 1 | Automatic lid actuation |
| Breadboard | 1 | Prototype circuit interconnection |
| Jumper wires |  | Electrical connections |
| USB cable | 1 | Arduino-PC connection and serial communication |
| Personal computer | 1 | PC-side serial processing and software system |
| Physical waste bin | 1 | Prototype enclosure and waste container |

Software resources used for the Arduino firmware, serial listener, backend/API, database and web dashboard are [INFORMATION REQUIRED]. These technologies should be inserted exactly as implemented.

## 3.3 Functional Requirements

| ID | Functional Requirement |
| --- | --- |
| FR-01 | The system shall detect a person approaching the bin using the person-detection ultrasonic sensor. |
| FR-02 | The system shall use an approximate 15 cm threshold to determine when to initiate automatic lid opening. |
| FR-03 | The system shall command the servo to move the lid to the open position of 180 degrees. |
| FR-04 | The system shall return the lid to the closed position of 90 degrees after approximately five seconds. |
| FR-05 | The system shall wait for the person to leave before accepting another opening trigger. |
| FR-06 | The system shall measure the distance from the fill-level sensor to the waste surface. |
| FR-07 | The system shall obtain seven readings for fill-level processing. |
| FR-08 | The system shall use a median-based method to reduce noisy fill-level measurements. |
| FR-09 | The system shall convert the filtered distance into a fill percentage using the calibrated empty and full distances. |
| FR-10 | The system shall constrain the calculated fill percentage to the range 0-100%. |
| FR-11 | The Arduino shall communicate sensor readings and system status to a PC through USB serial communication. |
| FR-12 | The software system shall process/store relevant information and present monitoring information through a web dashboard. |

## 3.4 Non-Functional Requirements

| ID | Category | Requirement |
| --- | --- | --- |
| NFR-01 | Usability | The monitoring interface should present system information in a clear form. |
| NFR-02 | Reliability | The control logic should behave consistently under valid operating conditions. |
| NFR-03 | Maintainability | Hardware pin assignments, calibration values and software modules should be organized so that changes can be made without redesigning the entire system. |
| NFR-04 | Performance | Person detection, lid control and fill-level processing should respond within the timing implemented by the prototype; no unsupported numerical benchmark is claimed. |
| NFR-05 | Accuracy | Fill-level estimation should use calibration and median-based filtering to reduce measurement instability; no unmeasured accuracy percentage is claimed. |
| NFR-06 | Security | Access to stored or dashboard data should be protected according to the implemented software controls. [INFORMATION REQUIRED: actual controls]. |
| NFR-07 | Scalability | The architecture should permit future extension, but the implemented prototype is not claimed to be a deployed multi-bin system. |
| NFR-08 | Availability | The monitoring system depends on the components required by the implemented deployment. [INFORMATION REQUIRED: network/server availability details]. |

# 4. SYSTEM DESIGN

## 4.1 Proposed System

InteliBin was designed as a layered embedded/software system. The physical layer consists of the waste bin, two ultrasonic sensors and a servo. The control layer is the Arduino Uno. Communication from the Arduino to the PC occurs through USB serial communication. The PC-side listener passes or transforms the information for the backend/API, which interacts with the database and dashboard.

[INSERT FIGURE: Physical Bin -> Ultrasonic Sensors + Servo -> Arduino Uno -> USB Serial Communication -> PC/Serial Listener -> Backend/API -> Database -> Web Dashboard.]

Figure 2: InteliBin Overall System Architecture.

## 4.2 Hardware Design

| Device/Signal | Arduino Pin | Function |
| --- | --- | --- |
| Person sensor TRIG | D9 | Triggers proximity measurement |
| Person sensor ECHO | D10 | Receives proximity echo pulse |
| Fill sensor TRIG | D4 | Triggers fill-level distance measurement |
| Fill sensor ECHO | D5 | Receives fill-level echo pulse |
| Servo signal | D6 | Controls lid position |

[INSERT FIGURE: Arduino Uno connected to two HC-SR04 sensors, servo motor, power/ground, breadboard and USB connection. Use the exact D9/D10, D4/D5 and D6 pin assignments.]

Figure 3: InteliBin Hardware Wiring Diagram.

## 4.3 Person Detection and Lid-Control Design

The person-detection sensor measures the distance to an object in front of the bin. When the measured distance is approximately 15 cm or less, the lid-control logic commands the servo to the open position of 180 degrees. The lid remains open for approximately five seconds and then moves to 90 degrees. After closing, the system does not accept another opening event until the person is no longer detected within the triggering condition.

[INSERT FIGURE: Start -> measure person distance -> valid and <=15 cm? -> open servo to 180 degrees -> wait about 5 s -> close to 90 degrees -> check whether person has left -> when clear, return to monitoring.]

Figure 4: Automatic Lid Control Flowchart.

## 4.4 Fill-Level Measurement Design

The fill-level sensor is mounted so that it points downward into the bin. The empty-bin reference distance is 20.9 cm and the full-bin reference distance is 4.0 cm. Seven readings are obtained for a fill-level measurement cycle. The readings are ordered and a median-based value is selected to reduce the influence of isolated noisy measurements.

| Calibration State | Distance |
| --- | --- |
| Empty bin | 20.9 cm |
| Full bin | 4.0 cm |

The filtered current distance is converted to a fill percentage using the following relationship:

Fill Percentage = ((Empty Distance - Current Distance) / (Empty Distance - Full Distance)) x 100

The result is constrained between 0% and 100%. Values below 0% are represented as 0%, while values above 100% are represented as 100%.

[INSERT FIGURE: Take seven distance readings -> validate readings -> sort/select median -> apply 20.9 cm empty and 4.0 cm full calibration -> calculate percentage -> constrain to 0-100% -> output/transmit value.]

Figure 5: Fill-Level Measurement Flowchart.

## 4.5 Software Architecture

The software architecture contains a PC/serial listener, backend/API, database and web dashboard. The Arduino is responsible for physical sensing and control, while the software side receives and presents system information. The exact language/framework used for each software component is [INFORMATION REQUIRED].

## 4.6 Data Flow

Data originates from the sensors. The Arduino processes proximity and fill-level measurements and controls the servo. Relevant readings and status information are sent over USB serial communication to the PC. PC-side software receives the serial information and integrates it with the backend/API. The backend interacts with the database and supplies information to the dashboard. The exact message format, update interval and API routes are [INFORMATION REQUIRED].

[INSERT FIGURE: Show sensor data entering Arduino; local lid-control output to servo; serial data to PC listener; backend processing; database storage; dashboard presentation.]

Figure 6: InteliBin Data Flow Diagram.

## 4.7 Database Design

A database is part of the completed software architecture, but the actual schema was not supplied with the thesis instructions. The final thesis must include the real entities, attributes, primary keys, foreign keys and relationships. No tables are invented in this draft.

[INSERT FIGURE: Insert the ER diagram generated from the actual implemented database. [INFORMATION REQUIRED].]

Figure 7: InteliBin Database Entity Relationship Diagram.

## 4.8 Dashboard Design

The web dashboard is intended to present useful monitoring information from the InteliBin system. The exact implemented pages, cards, charts, status indicators, authentication screens and historical views are [INFORMATION REQUIRED]. Only interfaces that exist in the completed project should be included as screenshots.

# 5. SYSTEM IMPLEMENTATION

## 5.1 Description of the Developed System

The completed InteliBin prototype integrates the physical waste bin with embedded electronics and a software monitoring system. The embedded portion performs person detection, automatic lid control and fill-level measurement. The software portion receives information from the Arduino through a PC-side serial connection and integrates it with a backend/API, database and dashboard.

## 5.2 Hardware Implementation

### 5.2.1 Arduino Uno

The Arduino Uno serves as the central embedded controller. It receives echo signals from the two HC-SR04 sensors, generates trigger signals, calculates distance values, applies the lid-control logic, calculates the fill level and controls the servo. It also provides serial communication to the connected PC through USB.

### 5.2.2 Person-Detection Sensor

The person-detection HC-SR04 uses D9 as TRIG and D10 as ECHO. Distance is measured repeatedly and compared with the approximate 15 cm detection threshold. A qualifying detection initiates the automatic lid sequence.

### 5.2.3 Fill-Level Sensor

The fill-level HC-SR04 uses D4 as TRIG and D5 as ECHO. It is oriented downward into the bin. The implementation collects seven readings, applies a median-based filtering step and converts the resulting distance using the calibrated empty and full references.

### 5.2.4 Servo and Lid Mechanism

The servo signal is connected to D6. The closed position is 90 degrees and the open position is 180 degrees. During an opening event, the servo moves to 180 degrees, remains open for approximately five seconds, and then returns to 90 degrees.

### 5.2.5 Physical Integration

The sensors, servo, Arduino, breadboard and wiring were integrated with the physical bin to form the prototype. Exact mounting dimensions, mechanical linkage details and photographs are [INFORMATION REQUIRED].

[INSERT FIGURE: Insert a clear photograph of the actual completed bin showing sensor placement, lid/servo mechanism and controller where visible.]

Figure 8: Completed InteliBin Prototype.

## 5.3 Embedded Software Implementation

### 5.3.1 Distance Measurement

Distance measurement is based on the ultrasonic time-of-flight principle. A trigger pulse causes the sensor to emit an ultrasonic burst. The echo duration represents the round-trip travel time. The firmware converts this time into distance. The exact Arduino source code should be included in Appendix A.

### 5.3.2 Person Detection and Leave Logic

The measured person distance is compared with the configured threshold. When the threshold condition is satisfied, the lid is opened. After the timed open period and closing action, the firmware requires the person to leave the triggering region before another opening event is permitted. This state-based behavior prevents continuous reopening caused by the same person remaining in front of the sensor.

### 5.3.3 Median Filtering

For fill-level measurement, seven readings are collected. A median-based process is applied so that the representative value is determined by the middle of the ordered measurements rather than by a single raw reading. This is particularly useful when an isolated measurement is inconsistent with the rest of the sample. No numerical noise-reduction percentage is claimed because such a statistic was not supplied.

### 5.3.4 Calibration and Fill Percentage

Calibration maps physical distance to the application-specific empty and full conditions. InteliBin uses 20.9 cm for empty and 4.0 cm for full. The difference between these values defines the usable fill-depth range. The current filtered distance is normalized within this range and multiplied by 100. The value is then constrained to the valid percentage range.

### 5.3.5 Serial Output

The Arduino provides sensor readings and system status to the PC through USB serial communication. The serial baud rate, exact field names, delimiters and example output messages are [INFORMATION REQUIRED]. These details should be documented from the actual firmware rather than assumed.

## 5.4 Software Integration

### 5.4.1 PC Serial Listener

The PC-side listener receives data sent by the Arduino. Its exact implementation language, serial library, parsing logic and error-handling behavior are [INFORMATION REQUIRED].

### 5.4.2 Backend/API

The backend/API receives or accesses data from the PC-side integration layer and supports the database/dashboard components. The actual framework, endpoints, request methods and payload formats are [INFORMATION REQUIRED].

### 5.4.3 Database

Relevant sensor/system information is stored in the project database. The database management system, table names, attributes and retention behavior are [INFORMATION REQUIRED].

### 5.4.4 Web Dashboard

The web dashboard presents monitoring information to the user. The exact implemented dashboard functions must be described from the completed system. [INFORMATION REQUIRED: provide dashboard screenshots or source files/pages].

[INSERT FIGURE: Insert an actual screenshot of the implemented dashboard.]

Figure 9: Web Dashboard.

[INSERT FIGURE: Insert only if this screen or dashboard component actually exists.]

Figure 10: Fill-Level Monitoring Interface.

[INSERT FIGURE: Insert only if this screen or dashboard component actually exists.]

Figure 11: Sensor/Lid Status Interface.

## 5.5 Complete System Integration

Hardware/software integration was completed successfully. The final system links sensor measurements and embedded control with PC-side communication and the software monitoring environment. This result verifies that the project moved beyond isolated hardware demonstrations to an integrated prototype.

# 6. TESTING AND VERIFICATION

## 6.1 Testing Methodology

Testing was organized at three levels: component testing, integration testing and complete-system testing. Component testing verified individual hardware and embedded functions. Integration testing verified communication and interaction between components. System testing considered the completed InteliBin chain. This structure reflects the actual incremental development process and makes it possible to relate test evidence directly to requirements.

## 6.2 Component Testing

| ID | Test | Input/Condition | Expected Result | Actual Result | Status |
| --- | --- | --- | --- | --- | --- |
| CT-01 | Servo movement | Command servo to closed/open positions | Servo moves between 90 and 180 degrees | Servo movement worked correctly | Pass |
| CT-02 | Person sensor distance | Place an object/person at different observable distances | Sensor produces distance readings | Person sensor detected distances correctly | Pass |
| CT-03 | Person threshold | Approach within approximately 15 cm | Detection condition becomes true | Detection triggered automatic lid opening | Pass |
| CT-04 | Lid timing/positions | Trigger an opening event | Open to 180 degrees, remain open ~5 s, close to 90 degrees | Observed as expected | Pass |
| CT-05 | Leave-before-retrigger logic | Remain within detection area after an opening event | System does not immediately accept another trigger until person leaves | System waited for the person to leave | Pass |
| CT-06 | Fill sensor distance | Place material/target beneath fill sensor | Sensor produces distance readings | Fill-level sensor produced distance readings | Pass |
| CT-07 | Calibration | Measure empty and full reference positions | References established | 20.9 cm empty; 4.0 cm full | Pass |
| CT-08 | Median filtering | Process seven fill-level readings | Median-based representative value used | Seven-reading median filtering implemented | Pass |

## 6.3 Integration Testing

| ID | Integration | Condition | Expected Result | Actual Result | Status |
| --- | --- | --- | --- | --- | --- |
| IT-01 | Person sensor + servo | Approach within threshold | Sensor event causes lid opening | Automatic lid opening succeeded | Pass |
| IT-02 | Fill sensor + Arduino | Obtain fill readings | Arduino receives/processes distance data | Complete hardware test succeeded | Pass |
| IT-03 | Arduino + PC | Connect Arduino by USB and run integration | PC receives serial sensor/status information | Hardware/software integration completed successfully | Pass |
| IT-04 | PC + backend | Transmit/process received data | Data reaches backend as implemented | [INFORMATION REQUIRED: evidence/result] | [INFORMATION REQUIRED] |
| IT-05 | Backend + database | Process data requiring persistence | Relevant data stored/retrieved | [INFORMATION REQUIRED: evidence/result] | [INFORMATION REQUIRED] |
| IT-06 | Backend + dashboard | Open dashboard with system operating | Monitoring information displayed | [INFORMATION REQUIRED: evidence/result] | [INFORMATION REQUIRED] |

## 6.4 System Testing

The complete hardware test succeeded, and hardware/software integration was subsequently completed successfully. This confirms that the core embedded functions operated together and that the embedded subsystem was connected to the software system. A final end-to-end dashboard test record should be inserted from the actual implementation evidence.

| ID | Requirement | Test Description | Expected Result | Actual Result | Status |
| --- | --- | --- | --- | --- | --- |
| ST-01 | Integrated hardware | Operate person detection, lid and fill sensing together | All hardware functions operate without preventing each other | Complete hardware test succeeded | Pass |
| ST-02 | Hardware/software integration | Run hardware while connected to software system | Sensor/status information is communicated into software chain | Integration completed successfully | Pass |
| ST-03 | End-to-end dashboard | Operate complete system and observe dashboard | Actual implemented dashboard reflects received/stored information | [INFORMATION REQUIRED] | [INFORMATION REQUIRED] |

## 6.5 Analysis of Test Results

The available test evidence demonstrates that the principal hardware requirements were achieved. The servo moved correctly between the configured positions. The person sensor produced usable distance readings and successfully triggered the automatic lid behavior. The lid opened to 180 degrees, remained open for approximately five seconds and returned to 90 degrees. The retrigger-prevention logic also worked by waiting for the person to leave.

The fill-level subsystem produced distance readings and was calibrated using the actual prototype's empty and full distances. Seven-reading median filtering was implemented. These results establish that the fill-level pipeline progressed from raw sensing through filtering and calibration.

The complete hardware test succeeded and hardware/software integration was completed successfully. However, this thesis does not invent numerical accuracy, latency, reliability or network-performance results. Such quantitative evaluation can only be added if actual measurements are provided.

## 6.6 Objective-Implementation-Verification Mapping

| Objective | Implemented Feature | Verification | Result |
| --- | --- | --- | --- |
| Person detection | HC-SR04 on D9/D10, ~15 cm threshold | CT-02, CT-03 | Implemented and passed |
| Automatic lid | Servo on D6; 180 open/90 closed; ~5 s | CT-04, IT-01 | Implemented and passed |
| Prevent repeated trigger | Person-leave logic | CT-05 | Implemented and passed |
| Fill-level measurement | HC-SR04 on D4/D5 | CT-06 | Implemented and passed |
| Calibration and percentage | 20.9 cm empty; 4.0 cm full; 0-100% | CT-07 | Implemented; calibration established |
| Noise reduction | Median of seven readings | CT-08 | Implemented |
| Arduino-PC communication | USB serial connection | IT-03 | Integration completed successfully |
| Backend/database/dashboard integration | Software chain | IT-04 to IT-06, ST-03 | Implementation stated as completed; detailed evidence [INFORMATION REQUIRED] |

# 7. CONCLUSION

## 7.1 Results and Achievements

The InteliBin project achieved the integration of sensing, actuation and software communication in a smart waste-bin prototype. Person proximity was detected using an ultrasonic sensor and used to trigger automatic lid opening. The servo successfully operated between the configured 90-degree closed and 180-degree open positions, with an approximately five-second open period. The system also implemented logic requiring the person to leave before another opening event.

Waste fill-level measurement was implemented using a second ultrasonic sensor. The system established empty and full calibration distances of 20.9 cm and 4.0 cm respectively, collected seven readings and applied median-based filtering before calculating a constrained 0-100% fill value.

The embedded hardware was integrated successfully, and hardware/software integration was subsequently completed. The resulting architecture connects the physical bin to an Arduino, PC-side serial communication, backend/API, database and web dashboard. The exact software implementation details remain to be inserted where marked.

## 7.2 Limitations and Problems Faced

Only limitations supported by the actual project experience should be presented as confirmed. The following possible limitations require confirmation before submission:

[CONFIRM WHETHER THIS APPLIES] The prototype depends on a PC/USB connection for communication with the software system.

[CONFIRM WHETHER THIS APPLIES] Ultrasonic readings can be influenced by the shape or orientation of waste surfaces.

[CONFIRM WHETHER THIS APPLIES] The system was tested primarily at prototype scale rather than through a large real-world deployment.

[CONFIRM WHETHER THIS APPLIES] The prototype requires an external power arrangement suitable for the Arduino, sensors and servo.

[INFORMATION REQUIRED: Add actual implementation problems encountered, such as wiring, mechanical, serial, backend or calibration issues.]

## 7.3 Future Work and Recommendations

The following items are potential future extensions rather than implemented InteliBin features. They should remain clearly separated from the completed system:

Standalone operation without dependence on a PC, if the current USB/PC dependency is confirmed.

Use of an IoT-capable controller such as an ESP32 for direct wireless communication.

Wireless or cloud-based monitoring.

Support for multiple bins from a single dashboard.

Notifications when configured fill thresholds are reached.

GPS/location information for distributed bins.

Battery or solar power for more independent deployment.

Improved sensing or sensor fusion for difficult waste surfaces.

Collection analytics and, at larger scale, predictive or route-optimization functions.

A mobile application if required by future users.

Larger-scale field deployment and longer-duration testing.

## 7.4 Conclusion

InteliBin demonstrates how embedded hardware and software can be combined to create a smart waste-bin prototype with both automatic physical interaction and digital monitoring capabilities. The project verified person detection, automatic lid control, calibrated fill-level sensing, median-based filtering, complete hardware operation and hardware/software integration. The work provides a foundation for future extensions toward more independent, networked and larger-scale smart waste monitoring while keeping the claims of the present thesis limited to functionality that was actually implemented and tested.

# 8. APPENDICES

## Appendix A: Arduino Source Code

[INSERT ACTUAL ARDUINO SOURCE CODE. Do not include passwords, API keys or credentials.]

## Appendix B: Hardware Wiring and Pin Configuration

| Component | Pin/Connection |
| --- | --- |
| Person HC-SR04 TRIG | Arduino D9 |
| Person HC-SR04 ECHO | Arduino D10 |
| Fill HC-SR04 TRIG | Arduino D4 |
| Fill HC-SR04 ECHO | Arduino D5 |
| Servo signal | Arduino D6 |
| Arduino-PC | USB |

## Appendix C: Software Installation and Deployment

[INFORMATION REQUIRED: actual installation steps for the serial listener, backend/API, database and dashboard.]

## Appendix D: Database Schema

[INFORMATION REQUIRED: insert the actual SQL schema or exported database structure.]

## Appendix E: API and Serial Communication Examples

[INFORMATION REQUIRED: baud rate, actual serial output examples, API routes and example request/response payloads.]

## Appendix F: Selected Backend Code

[INFORMATION REQUIRED: insert selected non-sensitive backend code relevant to sensor ingestion, storage and dashboard data access.]

## Appendix G: Additional Test Evidence

[INSERT ACTUAL photographs, serial-monitor output, screenshots, logs or recorded test observations.]

# 9. REFERENCES

Arduino. (n.d.). Arduino UNO R3. Arduino Documentation. https://docs.arduino.cc/hardware/uno-rev3

Kannan, D., Khademolqorani, S., Janatyan, N., & Alavi, S. (2024). Smart waste management 4.0: The transition from a systematic review to an integrated framework. Waste Management, 174, 1-14. https://doi.org/10.1016/j.wasman.2023.08.041

Pardini, K., Rodrigues, J. J. P. C., Kozlov, S. A., Kumar, N., & Furtado, V. (2020). IoT-based framework for smart waste monitoring and control system: A case study for smart cities. Engineering Proceedings, 2(1), 90.

SparkFun Electronics. (n.d.). HC-SR04 ultrasonic ranging module datasheet. https://cdn.sparkfun.com/assets/a/8/9/c/a/HC-SR04-33_Datasheet.pdf

United Nations Environment Programme (UNEP) & International Solid Waste Association (ISWA). (2024). Global Waste Management Outlook 2024: Beyond an Age of Waste - Turning Rubbish into a Resource. UNEP. https://doi.org/10.59117/20.500.11822/44939

Zoumpoulis, P., Konstantinidis, F. K., Tsimiklis, G., & Amditis, A. (2024). Smart bins for enhanced resource recovery and sustainable urban waste practices in smart cities: A systematic literature review. Cities, 152, 105150.

FINALIZATION NOTE: This is a complete thesis draft based on the project facts supplied. Every [INFORMATION REQUIRED] or [CONFIRM WHETHER THIS APPLIES] marker must be resolved with the actual project evidence before academic submission. No missing implementation details or test statistics were fabricated.